PROPOSED SURVEY PLATFORM MODULES

Working product name TBD - v9 platform planning

Goal: turn the current FieldBook Sync application into one module within a larger, project-centered surveying platform that can eventually serve as a one-stop shop for field data, control, calculations, QA/QC, GIS/CAD exchange, and deliverables.

Design principle: raw survey evidence is never silently changed. Adjustments, transformations, AI interpretations, cleanup, and corrections create traceable derived/revised data.

1. Core Platform Foundation

These are the pieces v9 should establish first so every later module uses the same project data, coordinate-system rules, evidence and revision model.

| Module | Purpose | Key capabilities | Priority |
| --- | --- | --- | --- |
| Project & Data Hub | The shared foundation for every job and every module. | Project/client/job metadata Shared point, observation, structure, file and attachment records Project tree, statuses, notes and workspace state Single source of truth across all modules | v9.0 Core |
| Coordinate Systems & Geodesy | Centralize CRS, datum, units and grid/ground logic. | State Plane, UTM and custom/local systems Horizontal/vertical datum, epoch and geoid metadata Grid-to-ground and combined scale factors Localization/calibration and distortion previews Multi-zone project support | v9.0 Core |
| Import / Export Framework | One consistent pipeline for survey and geospatial file exchange. | CSV/PNEZD, LandXML, DXF, SHP, GPKG, KML/KMZ, LAS/LAZ Trimble/Leica/Topcon/Carlson formats where practical Schema, units, CRS and provenance preservation Driver capability matrix and conversion reports | v9.0 Core |
| Revision & Audit System | Make professional calculations and edits reproducible. | Immutable raw observations/evidence Method, parameters, operator and timestamp tracking Revision comparison and rollback as new revisions Source-to-result lineage for calculations and reports | v9.0 Core |
| Map / Workspace | Provide a common spatial workspace shared by all survey tools. | Layer/project tree Map, selection, identify and review tools Properties/review panel Shared symbology, labels and visibility Module-aware tools without separate duplicate maps | v9.0 Core |

2. Survey Work Modules

These are the major production workflows that would sit on top of the v9 foundation.

| Module | Purpose | Key capabilities | Priority |
| --- | --- | --- | --- |
| FieldBook | Evolve the existing FieldBook Sync workflow into an integrated field-evidence module. | OCR/AI handwriting extraction Structure and point matching Dip recognition and invert support CNA/CNL statuses, photos and annotations Manual review with provenance | v9.0 First Module |
| Survey Control | Manage and validate control from GNSS and conventional observations. | OPUS, RTK base, VRS and conventional control Control averaging and weighted solutions Residuals, uncertainty and compatibility checks Network adjustment and least-squares direction Control reports and audit history | High |
| Traverse & Leveling | Handle conventional traverse and level-book calculations and QC. | Closed and constrained traverses Angular/linear closure and adjustment options Three-wire and differential leveling BS/FS balance, loop closure and bust detection Raw vs adjusted control reporting | High |
| COGO | Provide daily coordinate-geometry tools in the same project workspace. | Inverse and bearing-distance Intersections and offsets Station/offset and area Curves and later spiral support Translate, rotate, scale and best-fit tools | High |
| Boundary / Deed / Plat | Support record research, deed construction and boundary review. | Metes-and-bounds entry/extraction Record-vs-found comparison Closure analysis without forced closure Plat lot/block/easement construction Monument and legal-description workflows | High |
| Topo & Surface | Turn collected elevations and linework into useful topo products and QC. | Breaklines and TIN surfaces Contours, DEMs and spot elevations Profiles and cross sections Slope and cut/fill analysis Field-to-finish and elevation-bust QC | Medium |
| Utilities / Sewer | Expand the current sewer workflow into a general utility survey module. | Structures, dips, rims and inverts Pipe connectivity and slope/grade QC CNA/CNL and unresolved statuses Supplemental GIS comparison Crew completion maps and annotated field evidence | High |
| Stakeout / Crew Prep | Prepare controlled field packages and exports for crews. | Point-range allocation Stakeout lists and offsets Alignment/station information Cut/fill and descriptive notes KMZ/PDF/collector crew packages | Medium |
| Point Cloud / Drone | Support large survey datasets without trying to replace full photogrammetry software initially. | LAS/LAZ import and filtering Ground classification/thinning Surface extraction and profiles Volumes and comparison surfaces Photogrammetry handoff/integration | Later |

3. Integration, QA, Reporting & Expansion

These modules connect the production tools, enforce quality, generate deliverables and keep the platform maintainable.

| Module | Purpose | Key capabilities | Priority |
| --- | --- | --- | --- |
| CAD / GIS Workspace | Make the platform the survey intelligence layer between office systems. | ArcGIS Pro, QGIS, AutoCAD/Civil 3D and Global Mapper exchange Layer/schema mapping Coordinate-aware exports Reusable connection profiles Native-first with optional bridge tools | High |
| QA / QC Engine | Run consistent automated checks across every module. | Duplicate/conflicting IDs Rod-height and elevation busts Implausible slopes and grades CRS/datum/unit mismatches Closure, missing data and field-vs-office conflicts Company/client tolerance profiles | v9.0 Core |
| Reports & Deliverables | Create review-ready survey outputs from shared project data. | Control/traverse/level reports Annotated field books Coordinate lists and exhibits KMZ/GIS/CAD packages Project survey reports and PLS/PE signature areas | High |
| Automation / Company Rules | Let firms configure the software around their own standards. | Point-code and layer standards Tolerance/QC profiles Report templates Naming and export mappings Client/company-specific workflows | Medium |
| Collaboration | Support office/field review and team ownership as the platform grows. | Crew assignments and review status Comments and approvals Project sharing/sync Role permissions Notifications and conflict handling | Later |
| Plugin / Tool Framework | Keep specialized tools modular instead of growing one monolithic codebase. | Stable module/tool APIs Shared project-data access Extension registration and permissions Independent testing/versioning Future third-party or company-specific tools | v9 Foundation |

Planning note: Items marked v9.0 Core/Foundation are the structural pieces to establish first. The remaining modules can then be added incrementally without duplicating project data or survey logic.